# PWM Control of 12V Heating Element in the Incubator

We will need to add the ESP-IDF LEDC driver to the ESPIDF_Incubator project, since the 12V heating element can overheat too quickly and might present problems.

## AI Overview

Setting up PWM in ESP-IDF is typically done using the LEDC (LED Control) peripheral for general-purpose tasks (LEDs, servos) or the MCPWM (Motor Control) peripheral for advanced applications (motors, high-frequency signals). For standard projects, the LEDC API is the easiest and most common choice. [1, 2]

## Standard LEDC PWM Setup Steps

Setting up standard PWM involves configuring the Timer (frequency and resolution) and then assigning a Channel to that timer. [1]

### Include the header:

c

#include "driver/ledc.h"

Use code with caution.

- Configure the Timer:
Define the frequency, duty resolution, and the specific timer block.

c

ledc_timer_config_t ledc_timer = {

.speed_mode       = LEDC_LOW_SPEED_MODE,

.timer_num        = LEDC_TIMER_0,

.duty_resolution  = LEDC_TIMER_10_BIT, // 10-bit resolution (0 to 1023)

.freq_hz          = 5000,              // 5 kHz frequency

.clk_cfg          = LEDC_AUTO_CLK

};

ledc_timer_config(&ledc_timer);

Use code with caution.

- Configure the Channel:
Bind the channel to the timer and the GPIO pin you are using for output.

c

ledc_channel_config_t ledc_channel = {

.speed_mode     = LEDC_LOW_SPEED_MODE,

.channel        = LEDC_CHANNEL_0,

.timer_sel      = LEDC_TIMER_0,

.intr_type      = LEDC_INTR_DISABLE,

.gpio_num       = GPIO_NUM_2, // Adjust to your desired GPIO

.duty           = 0,          // Initial duty cycle (0 = OFF)

.hpoint         = 0

};

ledc_channel_config(&ledc_channel);

Use code with caution.

[1, 2, 3, 4, 5]

## Adjusting the Signal

After configuring, you can dynamically adjust the duty cycle or frequency: [1, 2]

- Set Duty Cycle: The maximum value depends on your resolution. For 10_BIT, it ranges from 0 to 1023.

c

ledc_set_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0, 512); // ~50% Duty Cycle

ledc_update_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0);

Use code with caution.

[1]

Alternative: MCPWM (Motor Control)

If you are controlling motors, using the MCPWM module is highly recommended to take advantage of advanced features like dead-time insertion and synchronization. [1, 2]

- Include the header: #include "driver/mcpwm.h"

- Core steps: Initialize the GPIO pins, initialize the MCPWM unit using mcpwm_init(), and set your required frequency and duty using mcpwm_set_duty(). [1, 2, 3, 4, 5]

For a deep dive into the official framework implementation and setup wizards, check out the Random Nerd Tutorials ESP-IDF LEDC Guide or the official Espressif LEDC Documentation. [1, 2]

LED Control (LEDC) - ESP32 - Technical Documents

Key Settings of LED PWM Controller's API. Note. For an initial setup, it is recommended to configure for the timers first (by call...

Espressif Systems

LED Control - ESP32 - — ESP-IDF Programming Guide release-v4.2 ...

Change PWM Duty Cycle Using Software To set the duty cycle, use the dedicated function ledc_set_duty() . After that, call ledc_upd...

YouTube·Hideo Pratama

ESP-IDF: ESP32 GPIO PWM with LEDC (LED Control)

Sep 19, 2025 — Open the ESP-IDF Espressif extension. Expand the “Advanced” menu. Click the “New Project Wizard” option. Choose the “Use ESP-IDF v...