# Asking DeepSeek about incorporating an I2C Scan tool in an ESPIDF VSCode application

I have both written and incorporated an ESP-IDF I2C Scanner, but it wasn’t readily available, so I decided to ask DeepSeek for help.

Here’s the details:

What is the method for incorporating the the esp-idf i2c scan tool into a ESP-IDF application using the ESP-IDF extension on vscode?

Read 12 web pages

There isn't a single built-in "tool" to activate with a click in the VS Code extension. Instead, you incorporate the I2C scanner by writing code as part of your ESP-IDF application. This involves initializing the I2C driver and writing a function to probe addresses on the bus .

The process generally follows these steps within your ESP-IDF project in VS Code:

### 1. Create and Set Up Your Project

If you haven't already, create an ESP-IDF project using the extension.

- Open the Command Palette (Ctrl+Shift+P).

- Run the command ESP-IDF: Create Project and follow the prompts to set up a new project.

### 2. Write the I2C Scanner Code

You will need to add code to your app_main() function (or a separate function) to perform the scan. The core logic is the same regardless of the VS Code extension.

Here is a breakdown of the typical steps and a complete example:

a. Include Necessary Headers

You'll need the I2C driver and FreeRTOS headers.

c

#include <stdio.h>

#include "driver/i2c.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

b. Define I2C Pins and Configuration

Define the GPIO pins for SDA and SCL on your board.

c

#define I2C_MASTER_NUM     0          // I2C port number

#define I2C_MASTER_SDA_IO  21         // GPIO number for SDA

#define I2C_MASTER_SCL_IO  22         // GPIO number for SCL

#define I2C_MASTER_FREQ_HZ 400000     // I2C clock frequency

c. Initialize the I2C Driver

Use the i2c_config_t structure and i2c_param_config() and i2c_driver_install() functions to set up the I2C bus. Important: For newer versions of ESP-IDF (v4.4 and later), it is recommended to initialize the i2c_config_t struct to zero to avoid potential issues with uninitialized members .

c

esp_err_t i2c_master_init(void) {

i2c_config_t conf = {

.mode = I2C_MODE_MASTER,

.sda_io_num = I2C_MASTER_SDA_IO,

.scl_io_num = I2C_MASTER_SCL_IO,

.sda_pullup_en = GPIO_PULLUP_ENABLE,

.scl_pullup_en = GPIO_PULLUP_ENABLE,

.master.clk_speed = I2C_MASTER_FREQ_HZ,

};

// Zero the structure to avoid issues with new members

// i2c_config_t conf = { 0 }; // Alternatively, use this initialization

i2c_param_config(I2C_MASTER_NUM, &conf);

return i2c_driver_install(I2C_MASTER_NUM, conf.mode, 0, 0, 0);

}

d. Implement the Scan Function

This function iterates through all possible 7-bit I2C addresses (from 1 to 127) and attempts to communicate with a device . If it receives an acknowledgment (ACK), a device is present at that address.

c

static void i2c_scanner_task(void *pvParameters) {

int port = I2C_MASTER_NUM;

printf("Starting I2C scanner on port %d...\n", port);

while (1) {

printf("\n     0  1  2  3  4  5  6  7  8  9  a  b  c  d  e  f\n");

for (int i = 0; i < 128; i += 16) {

printf("%02x: ", i);

for (int j = 0; j < 16; j++) {

fflush(stdout);

uint8_t address = i + j;

if (address == 0) continue; // Skip general call address

i2c_cmd_handle_t cmd = i2c_cmd_link_create();

i2c_master_start(cmd);

i2c_master_write_byte(cmd, (address << 1) | I2C_MASTER_WRITE, true);

i2c_master_stop(cmd);

esp_err_t ret = i2c_master_cmd_begin(port, cmd, 50 / portTICK_PERIOD_MS);

i2c_cmd_link_delete(cmd);

if (ret == ESP_OK) {

printf("%02x ", address);

} else {

printf("-- ");

}

}

printf("\n");

}

printf("Scan complete. Waiting 5 seconds...\n");

vTaskDelay(5000 / portTICK_PERIOD_MS);

}

}

### 3. Integrate into app_main()

Call your initialization and create the scanner task in app_main().

c

void app_main(void) {

ESP_ERROR_CHECK(i2c_master_init());

printf("I2C driver initialized. Starting scanner task.\n");

xTaskCreate(i2c_scanner_task, "i2c_scanner", 4096, NULL, 5, NULL);

}

### 4. Build, Flash, and Monitor

This is where the VS Code extension shines.

- Press Ctrl+Shift+P to open the Command Palette.

- Run ESP-IDF: Build, Flash and Start a Monitor to compile your code, flash it to your ESP32, and open the serial monitor to see the scan results .

### Alternative: Using a Component from the Registry

Instead of writing the code yourself, you can use the ESP Component Registry . This is the most convenient method if you prefer a ready-made solution.

- In VS Code, open the Command Palette (Ctrl+Shift+P).

- Run the command ESP-IDF: Show ESP Component Registry .

- Search for "i2c scanner" or "i2cdev". Components like eil/i2cdev or esp-idf-lib/i2cdev offer an example scanner that can be integrated into your project .

- Click the Install button for the component. This will download it into your project's components/ folder.

- You can then use the examples provided by the component in your code.

### Important Note on GPIO Pins

The example code uses GPIO21 (SDA) and GPIO22 (SCL). This is a common configuration for many ESP32 development boards. However, you must verify the correct pins for your specific hardware and update the I2C_MASTER_SDA_IO and I2C_MASTER_SCL_IO definitions accordingly. If the pins are wrong, the scanner will not find any devices.

**** End of this chat ****

So, I decided to try the Alternative method first, since that looked like the easiest way to get the job done. The component sources for the i2c scanner involved a bit of work after all, and I was hesitant because I wanted to steer clear of any additional burden for incorporating from a third party source. As a result, I decided to follow the first method. I already had incorporated several I2C device components anyways, so some of the fundamental I2C foundation was already present.

So, I could see that the Scan function was exactly the key, and I copied in the block of code into my application. Right away, I had feedback from the vscode environment, so I had to now tell DeepSeek my observations and what it would advise.

Here’s the chat:

There does seem to be some modifications needed, since i am getting a warning from the Intellisense editor extension about "i2c_cmd_handle_t" not being found. I am using the newer "driver/i2c_master.h"

You're absolutely right! In ESP-IDF v5.0 and later, the I2C API was completely redesigned. The old driver/i2c.h (with i2c_cmd_handle_t) is deprecated, and you should use the new driver/i2c_master.h API. This explains why IntelliSense can't find i2c_cmd_handle_t—it no longer exists in the new API.

Here's the complete updated scanner code using the new i2c_master.h API:

### Complete I2C Scanner for ESP-IDF v5.0+

1. Updated Headers

c

#include <stdio.h>

#include "driver/i2c_master.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "esp_log.h"

2. Define I2C Configuration

c

#define I2C_MASTER_NUM      0          // I2C port number

#define I2C_MASTER_SDA_IO   21         // GPIO for SDA

#define I2C_MASTER_SCL_IO   22         // GPIO for SCL

#define I2C_MASTER_FREQ_HZ  400000     // 400 kHz

static const char *TAG = "I2C_SCANNER";

3. Initialize the I2C Bus (New API)

The new API uses i2c_master_bus_handle_t and i2c_master_bus_config_t:

c

i2c_master_bus_handle_t bus_handle;

esp_err_t i2c_master_init(void) {

i2c_master_bus_config_t bus_config = {

.clk_source = I2C_CLK_SRC_DEFAULT,

.i2c_port = I2C_MASTER_NUM,

.scl_io_num = I2C_MASTER_SCL_IO,

.sda_io_num = I2C_MASTER_SDA_IO,

.glitch_ignore_cnt = 7,

.flags.enable_internal_pullup = true,

};

return i2c_new_master_bus(&bus_config, &bus_handle);

}

4. Updated Scanner Function (New API)

The new API uses i2c_master_probe() which is much simpler than the old command-based approach:

c

static void i2c_scanner_task(void *pvParameters) {

ESP_LOGI(TAG, "Starting I2C scanner on bus %d...", I2C_MASTER_NUM);

while (1) {

ESP_LOGI(TAG, "\n     0  1  2  3  4  5  6  7  8  9  a  b  c  d  e  f");

int device_count = 0;

for (int i = 0; i < 128; i += 16) {

printf("%02x: ", i);

for (int j = 0; j < 16; j++) {

uint8_t address = i + j;

if (address == 0) {

printf("-- ");

continue;

}

// Probe the address with a 50ms timeout

esp_err_t ret = i2c_master_probe(bus_handle, address, 50);

if (ret == ESP_OK) {

printf("%02x ", address);

device_count++;

} else {

printf("-- ");

}

fflush(stdout);

}

printf("\n");

}

ESP_LOGI(TAG, "Scan complete. Found %d device(s). Waiting 5 seconds...\n", device_count);

vTaskDelay(5000 / portTICK_PERIOD_MS);

}

}

5. Updated app_main()

c

void app_main(void) {

ESP_ERROR_CHECK(i2c_master_init());

ESP_LOGI(TAG, "I2C driver initialized. Starting scanner task.");

xTaskCreate(i2c_scanner_task, "i2c_scanner", 4096, NULL, 5, NULL);

}

### Alternative: Using a Device Handle (More Advanced)

If you want to go further and actually communicate with a detected device, the new API uses device handles:

c

// After detecting a device, you can create a device handle

i2c_master_dev_handle_t dev_handle;

i2c_device_config_t dev_config = {

.dev_addr_length = I2C_ADDR_BIT_LEN_7,

.device_address = 0x3C,  // Example address

.scl_speed_hz = 400000,

};

ESP_ERROR_CHECK(i2c_master_bus_add_device(bus_handle, &dev_config, &dev_handle));

// Then read/write using:

// i2c_master_transmit(dev_handle, tx_buffer, len, timeout)

// i2c_master_receive(dev_handle, rx_buffer, len, timeout)

// i2c_master_transmit_receive(dev_handle, tx_buffer, tx_len, rx_buffer, rx_len, timeout)

### CMakeLists.txt Changes

Make sure your CMakeLists.txt includes the I2C driver component. Usually it's included automatically, but if you have issues:

cmake

idf_component_register(SRCS "main.c"

INCLUDE_DIRS ".")

### Key Differences from the Old API

| Old API (deprecated) | New API (v5.0+) |
| --- | --- |
| #include "driver/i2c.h" | #include "driver/i2c_master.h" |
| i2c_cmd_handle_t | No equivalent - uses i2c_master_bus_handle_t |
| i2c_param_config() + i2c_driver_install() | i2c_new_master_bus() |
| i2c_master_start()/write_byte()/stop() | i2c_master_probe() for scanning |
| Command links (i2c_cmd_link_create()) | Direct functions: i2c_master_transmit(), i2c_master_receive() |

### Troubleshooting IntelliSense Warnings

If IntelliSense still shows warnings after updating:

- Make sure your ESP-IDF version is v5.0 or newer:

bash

idf.py --version

- Update the ESP-IDF extension's Python environment:

  - Open Command Palette (Ctrl+Shift+P)

  - Run ESP-IDF: Set ESP-IDF Target

  - Run ESP-IDF: Install ESP-IDF Python Packages

- Regenerate compile_commands.json:

bash

idf.py reconfigure

This helps IntelliSense understand your project structure.

- Restart VS Code after making changes to clear any cached IntelliSense data.

The new API is much more intuitive and less error-prone, and i2c_master_probe() handles all the START/STOP conditions and ACK/NACK detection for you automatically!